Installation Guide

For

ISA ESS Drivers

How to identify what ESS sound card you have?

- Open “Control Panel” then double click on “System” icon.  This will take you to “System Properties”

- Click on “Device Manager” tab.  Then scroll down to “Sound, Video, and Game Controller”.  Click on plus sign to display the items below it.   This will tell you what ESS sound card you have.

Before Installation:

- Be sure that you download correct driver for your sound card and audio driver un-installation programs for all ISA sound cards.

- Unzip the ESS driver and audio driver un-installation programs to C:\temp or C:\tmp or A:\ using unzips programs such as WinZip.   If you do not have WinZip programs.  You can download latest version at www.winzip.com.   Below is the instruction how to install WinZip after you download.

Windows95/98:

- Click on “Start, Run” you can type in “c:\temp\setup.exe or c:\tmp\setup.exe or a:\setup.exe” then click “OK”.  Or you can click on “Browse” button to find where you unzip the driver and un-installation programs.

- The programs will prompt, “Welcome to setup programs.  This program will uninstall ESS driver on your computer”.  Click “Next”, programs will then prompt “Are you sure you want to uninstall the driver?”  Click “Yes”, this will automatic uninstall all the ESS drivers in your computer.

- The programs will prompt, “Setup program has finished removing all the ESS drivers from you computer” “Do you want to restart your computer?”   Click “Yes, I want to restart my system now” then click on “Finish” to restart the system.

- During boot up, system will prompt “Add New Hardware Wizard” click “Next”

Window95

    - If you unzip ESS drivers to diskette.  Then insert the disk and click “Next”.  The system will go through and search for driver in your diskette.  If you unzip ESS driver to c:\temp or c:\tmp.  Windows will bypass search in a:\.  Windows display its cannot found driver.

    - Click on “Other Location” button.  Type the location where unzip the driver.  Process follows the rest instruction by windows.  If windows prompt that it cannot found driver.  Always browse to location where you unzip the driver.

- If the system prompt for files “mjstick.drv”.  Type in c:\windows\system and click “OK”.  This will install the joystick port automatic.

Windows98

- System will prompt, “What do you Windows to do?” the default is “Search for better driver for your device (Recommended).”  Select the default.

- If you unzip ESS driver to diskette. Then check  “Floppy disk device” and uncheck all the rest.  If you unzip to c:\temp or c:\tmp.  Then check “Specify a location” and uncheck all rest and type in the location where you unzip the driver.

- Then click “Next”.  System has found the driver.  Click “Next”, system will automatic install the driver.  When driver install finish click “Finish”.    System will go through install all rest drivers automatic.

- If the system prompt for files “mjstick.drv”.  Type in c:\windows\system and click “OK”.  This will install the joystick port automatic.

- Windows will now boot to desktop.  You are finish.  You have the optional to reboot the system again.